Geometry

Add Material  —  Creates material and merges it with the active bodies in the model, effectively creating a single part.

Add Frozen  —  Creates material, but adds it to the model as frozen bodies, without merging them with other bodies in the model. Adds the new material as a separate, independent body, even if it overlaps with existing bodies.

Form New Part tool combines selected bodies into a single part, enabling shared topology for meshing and analysis. This feature is accessed through the context menu (right-click on the graphics screen) or the Tools menu, after selecting the desired bodies.

Share Topology is a crucial step for ensuring a conformal mesh across multiple bodies or parts, especially when dealing with fluid-solid interactions or complex geometries. It ensures a continuous mesh at interfaces, preventing gaps and non-matching meshes that can lead to inaccuracies or errors.

Meshing

Capture curvature:

Mesh sizing for capturing curvature involves refining the mesh in areas of high curvature to accurately represent the geometry. This is achieved by adjusting parameters like minimum/maximum element size, growth rate, and curvature normal angle. In ANSYS meshing, setting "Capture Curvature" to "No" means the mesh will not be refined to specifically capture sharp geometric features like curves and edges. This is often done to reduce computational cost, especially in models where curvature details are not crucial for the simulation's accuracy.

Inflation:

Inflation is primarily used to create a fine mesh, or layers of fine mesh, in areas of interest, particularly for resolving boundary layers. The inflation growth rate controls how the size of elements increases in the inflation layers added near boundaries. A higher growth rate means a faster increase in element size with each successive layer, while a lower rate means a more gradual increase.

Face meshing:

Face meshing in ANSYS Fluent allows for the generation of structured (mapped) or free meshes on selected faces. It's a technique used to create a more uniform and high-quality mesh, particularly on faces that are approximately rectangular. This can be crucial for achieving accurate simulation results, especially in areas with complex geometries or where a structured mesh is desired.

Mesh Quality:

Orthogonal Quality: Measures the alignment between vectors normal to cell faces and vectors connecting cell centroids. Values closer to 1 indicate better orthogonality, while values closer to 0 indicate poor orthogonality.

Skewness: Quantifies how much a cell deviates from an ideal shape (e.g., a cube or tetrahedron). Higher skewness values indicate more distorted cells. A maximum skewness value below 0.95 is generally recommended for optimal results

Aspect Ratio: The ratio of the longest to the shortest edge of a cell. A low aspect ratio (close to 1) is desirable, indicating a well-proportioned cell.

Warping Factor: Relevant for quadrilateral faces, it measures the deviation of the face from planarity. A warping factor of 0 is ideal.

Setup

Single Precision (32-bit):

Uses 32 bits to represent a floating-point number. This is generally sufficient for many CFD simulations, but it has limitations in terms of accuracy and range.

Double Precision (64-bit):

Uses 64 bits to represent a floating-point number, providing higher accuracy and a wider range of values.

Solver process:

The solver process number refers to the number of parallel processes or threads that are used to solve the computational fluid dynamics (CFD) problem. These processes work together to divide the computational domain and solve it simultaneously, which can significantly reduce the overall simulation time, especially for large and complex models.

Parallel Processing:

ANSYS Fluent leverages parallel processing to speed up simulations. This involves dividing the computational domain (the mesh) into smaller partitions and assigning each partition to a separate process or thread.

Number of Processes:

The number of processes used is determined by the user and is typically limited by the number of CPU cores available on the system or the available licenses.

Pressure-based and density-based solvers:

Pressure-based and density-based solvers differ in the way how the continuity, momentum, and energy and species equations are solved. Historically speaking, the pressure-based approach was developed for low-speed incompressible flows, while the density-based approach was mainly used for high-speed compressible flows. However, recently both methods have been extended and reformulated to solve and operate for a wide range of flow conditions beyond their traditional or original intent. Pressure-based technology solves the pressure equation to conserve mass, and can be used in any flow simulation except with the following features: non-reflecting boundary conditions, and wet steam multiphase model.

Viscous models:

The k-epsilon, k-omega, and Spalart-Allmaras models are all used in computational fluid dynamics (CFD) to simulate turbulence. The k-epsilon model is a widely used, robust model for general turbulent flows, while the k-omega model is often preferred for flows with adverse pressure gradients or near walls due to its better near-wall behavior. The Spalart-Allmaras model is a simpler, one-equation model particularly suited for aerodynamic applications, especially in wall-bounded flows.

- k-epsilon Model:

A two-equation model that solves for turbulent kinetic energy (k) and its dissipation rate (epsilon). Well-established, robust, and computationally efficient for many industrial and aerospace applications. Can be inaccurate in flows with strong pressure gradients, separation, or complex geometries. General turbulent flows, internal flows, and flows where near-wall behavior is not critical.

- Standard k-ε model

- Oldest and simplest version.

- Good for simple flows (like pipe flow, flat plates).

- Stable and fast, but not very accurate for complex flows (like swirling, curved surfaces, or sudden changes in flow).

- Used in many industrial applications because it’s easy to solve.

- RNG k-ε model (Renormalization Group)

- Improved version of the standard model.

- Handles rotating, swirling, and rapidly changing flows better.

- Has extra math to adjust how ε behaves in different flow regions.

- More accurate than standard k-ε, but slightly more computationally expensive.

- Realizable k-ε model

- "Realizable" means it follows some mathematical constraints better, making it more physically accurate.

- Great for flows with strong curvature, separation, or recirculation (e.g., flow over a car or building).

- Has a different formula for ε, improving results in many practical problems.

Often preferred for external aerodynamics or complex internal flows.

- k-omega Model:

A two-equation model that solves for turbulent kinetic energy (k) and the specific dissipation rate (omega). More accurate near walls and in flows with adverse pressure gradients or separation compared to k-epsilon. Can be sensitive to inlet conditions and may require careful selection of the inlet turbulence parameters. Flows with adverse pressure gradients, separated flows, and flows where accurate near-wall behavior is important, such as in aerodynamics. SST (Shear Stress Transport) k-omega model combines the advantages of both k-epsilon and k-omega models by using k-omega near the wall and k-epsilon in the free stream.

- Spalart-Allmaras Model:

A one-equation model that solves for a modified turbulent viscosity. Simpler than two-equation models, computationally efficient, and particularly well-suited for aerodynamic simulations of wall-bounded flows and turbomachinery applications. May not be as accurate as k-omega for complex flows with strong separation or recirculation. Aerodynamic simulations, turbomachinery, and wall-bounded flows where computational efficiency is a priority.

Wall Functions:

| Type | Description | Mesh Requirement | Accuracy |
| --- | --- | --- | --- |
| 1. Standard Wall Functions | Uses formulas to approximate flow near wall instead of resolving it. | Coarser mesh (y+ > 30) | OK for simple flows |
| 2. Enhanced Wall Treatment | Blends between wall functions and full resolution depending on mesh. | Can use fine or coarse mesh | Good for complex walls |
| 3. Low-Re (or Two-Layer) Model | Resolves the entire boundary layer by solving all the way to the wall. | Very fine mesh (y+ ≈ 1) | Most accurate |

Turbulent Intensity (TI):

Turbulent intensity measures how strong the turbulence is in a flow compared to the average speed.

Where:

- u′ = root-mean-square of turbulent velocity fluctuations

- U = mean flow velocity

:

| Flow Type | Typical Values of Turbulent Intensity (%) |
| --- | --- |
| Very smooth pipe flow | 0.5 – 1% |
| Typical pipe/duct flow | 3 – 10% |
| Highly turbulent flow | 10 – 20%+ |

In ANSYS Fluent, if you're unsure, you can use a formula:

Where Re = Reynolds number.

Hydraulic Diameter (Dh):

Hydraulic diameter is used to define the "effective diameter" of non-circular ducts or pipes, especially in internal flows.

Formula:

- For a circular pipe: Dh=actual diameter

- For a rectangular duct:

where w = width, h = height

Solution

Solution methods:

| Method | Full Form | Best For | Key Idea | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| SIMPLE | Semi-Implicit Method for Pressure-Linked Equations | Steady flows | Solves pressure and velocity step-by-step | Stable, widely used | Converges slowly |
| SIMPLEC | SIMPLE-Consistent | Steady flows (faster) | Modified version of SIMPLE | Faster than SIMPLE | May be less stable in some cases |
| PISO | Pressure-Implicit with Splitting of Operators | Unsteady (transient) flows | Uses two correction steps for pressure-velocity | Accurate and fast for time-dependent problems | Uses more memory |
| Coupled | - | Compressible & high-speed flows | Solves pressure and velocity together in one loop | Very fast and accurate | Needs more RAM and CPU |

First and second order: When the flow is aligned with the mesh (e.g., laminar flow in a rectangular duct modeled with a quadrilateral or hexahedral mesh) the first-order upwind discretization may be acceptable. When the flow is not aligned with the mesh (i.e., when it crosses the mesh lines obliquely), however, first-order convective discretization increases the numerical discretization error (numerical diffusion). For triangular and tetrahedral meshes, since the flow is never aligned with the mesh, you will generally obtain more accurate results by using the second-order discretization.

| Term | Used For | Description | Where Found |
| --- | --- | --- | --- |
| Second Order | Pressure | Discretizes the pressure correction equation more accurately (e.g., in SIMPLE, PISO) | Under Discretization → Pressure |
| Second Order Upwind | Momentum, Energy, Turbulence, Species | Discretizes convective (advection) terms in transport equations | Under Discretization → Momentum / Energy / Turbulence, etc. |

In CFD (Computational Fluid Dynamics), "upwind" refers to a method of discretizing the convective (advection) terms in transport equations based on the direction of the flow.

Think of it like this:

“What’s happening upstream (in the direction the fluid came from) influences the downstream cell.”

| Method | Time Step Dependency | Stability | Speed per Step | Total Simulation Time | Usage in ANSYS |
| --- | --- | --- | --- | --- | --- |
| Implicit | Solves equations using unknown future values | Very Stable | Slower | Faster overall | Common in Fluent |
| Explicit | Solves using only known current/past values | Less Stable | Faster | Needs tiny time steps | Rare in Fluent (e.g. VOF) |

Green-Gauss node-based gradient schemes are a method used in computational fluid dynamics (CFD) to calculate the gradient of a variable at a node (point) in a mesh, particularly on unstructured grids. It leverages the divergence theorem to approximate the gradient, and is known for its ability to reconstruct accurate values of a linear function at a node, even on irregular meshes. This scheme is often preferred for its accuracy, especially on unstructured meshes, and is commonly used in finite volume methods.

Solution initialization:

Solution initialization assigns initial values to flow variables across the computational domain before the solver begins iterative calculations. This step is crucial for a successful simulation as the closer the initial guess is to the final solution, the faster the convergence.

Standard initialization: Use when you have specific initial conditions that you want to apply or when the flow is relatively simple.

Hybrid initialization: Use when you want a more automated approach to setting initial conditions, especially for complex flow scenarios, or when you don't have precise initial values.

Patching:

Patching is the process of initializing or modifying solution variables within specific cell zones or registers. It allows users to set initial conditions, introduce changes to the flow field, or initialize specific regions with desired values before starting or during a simulation.

Iterations:

Iterations refer to the repeated solution of governing equations to refine an approximate solution until it converges to a desired level of accuracy. Each iteration involves recalculating flow variables (like pressure, velocity, temperature) based on updated values from the previous iteration, gradually approaching the final, stable solution.

Scaled residuals:

Scaled residuals are used to assess the convergence of a numerical simulation. They represent the imbalances in the governing equations, scaled by a characteristic flux or quantity, making them a more reliable indicator of convergence than raw, unscaled residuals, especially for complex geometries and flow conditions.  A decreasing trend in scaled residuals over iterations is generally a good sign, indicating that the solution is approaching a converged state.